WINDOW CLEANING ROBOTS

Illustrated engineering and procurement report • 2026 edition

Five representative commercial/home window-cleaning robots

ECOVACS WINBOT W2 PRO OMNI • HOBOT S7 PRO • HOBOT-2S • HUTT DDC55 PRO • DREAME C1

Executive Summary

Window-cleaning robots are generally wall-climbing cleaning machines that combine negative-pressure adhesion, traction, wet or dry wiping, edge detection and a tether/backup power system. The market has moved from simple corded vacuum cleaners toward products with route planning, adaptive pressure control, mobile apps and portable stations.

| Robot | Observed price | Core differentiator | Position |
| --- | --- | --- | --- |
| ECOVACS WINBOT W2 PRO OMNI | $499.99 US list/current official price; €449 on ECOVACS Germany | Portable station + WIN-SLAM 4.0 | Top overall |
| HOBOT S7 PRO | Observed 2026 market price about €322–€399; price varies by region | 600-stroke/min dual mop + ultrasonic mist | Specialized scrubbing |
| HOBOT-2S | $299 official US sale price (listed from $349) | Ultrasonic spray + mature corded design | Best mature-value option |
| HUTT DDC55 PRO | Observed around €189–€200 / about CHF 180 at launch; regional pricing varies | Low-cost dual-disk architecture | Budget / broad surfaces |
| DREAME C1 | Observed €199 promotional price in Italy; €399 stated recommended retail price | CornerClean + Z/N planning | Strong value |

Bottom line: ECOVACS W2 PRO OMNI is the most complete workflow-oriented system in this set. Dreame C1 is especially attractive when corner coverage and promotional pricing matter. HOBOT-2S remains a mature ultrasonic-spray design with a relatively low purchase price, while HUTT DDC55 PRO emphasizes value. HOBOT S7 PRO is technically distinctive but its independent 2026 test results indicate slower cleaning, high noise and a short emergency mode relative to newer station-equipped competitors.

Scope, Method and Data Quality

Products covered: the five models carried forward from the requested shortlist.

Specifications are taken primarily from manufacturer pages; current prices are market observations and can change by country, retailer, tax and promotion.

Where independent 2026 testing conflicts with marketing claims, both are identified rather than silently averaged.

Exploded views are functional architecture diagrams, not proprietary internal CAD drawings.

Performance claims such as cleaning coverage, noise and battery runtime should be treated as manufacturer-test conditions unless independently measured.

For commercial/high-rise deployment, site-specific risk assessment is required; consumer robot safety claims are not equivalent to a certified façade-access system.

Price convention

Prices are shown in the currency reported by the source when a reliable current figure was available. No artificial currency conversion is used in the core comparison because exchange rates and regional retail pricing change.

How a Window-Cleaning Robot Works

Generic functional architecture shared by most modern glass-climbing cleaning robots.

Adhesion Physics and Fall Protection

Negative pressure creates a normal force; control electronics regulate adhesion while independent safety layers limit fall risk.

Navigation and Cleaning Paths

Typical Z/N/edge patterns. Actual route logic varies by model and sensor suite.

Operating Mechanism: Adhesion, Traction and Cleaning

A window robot has to solve three coupled mechanical problems: stay attached, move without slipping, and transfer enough mechanical/chemical energy into the glass to remove contamination. Negative pressure provides the attachment force, while wheels, belts or rotating mop assemblies create tangential traction. A cleaning pad then applies normal pressure and shear to the glass. Water or cleaning solution reduces surface energy and helps suspend contaminants.

| Subsystem | Engineering role | Typical failure mode | Design response |
| --- | --- | --- | --- |
| Vacuum blower | Generate negative pressure | Leak, blocked intake, loss of power | Pressure sensing, compensation, UPS, tether |
| Traction | Convert adhesion into motion | Wet slip / dirty tracks | High-friction drive, pressure control |
| Cleaning pad | Apply shear and absorb liquid | Saturation, streaks, wear | Washable microfiber, replaceable pads |
| Spray | Wet and loosen contamination | Clogging / over-wetting | Ultrasonic or pulse spray, metering |
| Edge sensing | Detect pane boundary | False edge / frame geometry | Optical/pressure sensors + route correction |
| Controller | Coordinate all functions | Localization or sensor error | Fallback route, return-to-start, voice/app alerts |

Safety Architecture

Primary adhesion: vacuum blower and sealed contact region.

Pressure compensation: dynamically increases or stabilizes suction when leakage or surface variation is detected.

Edge detection: detects the end of usable glass before the robot drives beyond the pane.

Emergency power: a small battery maintains suction long enough for the robot to remain attached or for an operator to intervene.

Physical tether: a separate load path limits consequences if adhesion is lost.

Station anchoring: portable stations such as the ECOVACS and Dreame Station designs can add a second level of restraint.

Software fallback: low-battery return, route resume and status alarms reduce operator workload but do not eliminate supervision.

Important engineering note

For exterior high-rise use, the tether must be treated as life-safety equipment only when its anchorage, load rating, inspection and operating procedure are appropriate to the actual installation. A consumer robot should not be treated as a substitute for a certified rope-access or façade-maintenance system.

Cross-Model Specification Comparison

| Model | Robot size | Weight | Adhesion / suction | Backup / battery | Navigation |
| --- | --- | --- | --- | --- | --- |
| ECOVACS WINBOT W2 PRO OMNI | 271 × 271 × 77 mm | 1.6 kg robot; 5.5 kg station | 5,500 Pa max (2,800 Pa while moving) | 4,500 mAh; up to 110 min; 30 min power-loss hold | WIN-SLAM 4.0; frame/edge detection; 7 modes |
| HOBOT S7 PRO | 262 × 264 × 89 mm | about 1.7–3.0 kg depending on source/market configuration | 4,800 Pa | about 20 min normal backup; ~26–27 min measured emergency hold in 2026 test | AI / gyro-assisted route planning; Z/N patterns; app + remote |
| HOBOT-2S | 240 × 240 × 86 mm | 1.3 kg | 6.0 kgf adhesive force (manufacturer specification) | 20 min UPS / emergency hold | AI Technology V4; gyro; four edge-leakage sensors; zigzag / N-way |
| HUTT DDC55 PRO | compact body; roughly 1–1.1 kg in current listings | about 1.1 kg in a 2026 commercial listing | 5,400 Pa manufacturer claim | manufacturer/market listings report emergency retention around 30 min | HUTTHINK variable-frequency algorithm; AI sensors; obstacle avoidance |
| DREAME C1 | 270 × 270 × 80.5 mm | 1.6 kg | 5,500 Pa max; 3,300 Pa maintained during normal operation | corded robot; C1 Station variant has 7,800 mAh / up to 180 min | AI path planning; Z/N selection; >10 sensors; edge detection |

| Model | Spray / tank | Modes | App | Station | Typical use |
| --- | --- | --- | --- | --- | --- |
| W2 PRO OMNI | 6 nozzles / 60 ml | 7 | Yes | Yes | Large / hard-to-reach glass |
| S7 PRO | Ultrasonic / ~52 ml test | Several | Yes | No | Active scrubbing |
| HOBOT-2S | 2×26 ml ultrasonic | 3 | Yes | No | Home / office |
| DDC55 PRO | Pulse / 80 ml | Multiple | Remote | No | Budget / broad surfaces |
| Dreame C1 | Dual / 80 ml | 5 | Yes | C1 Station optional | Corner-focused cleaning |

Procurement Decision Framework

| Priority | Best fit | Why |
| --- | --- | --- |
| Maximum convenience | W2 PRO OMNI | Portable station, battery operation, automatic cable management |
| Corner coverage | Dreame C1 | CornerClean four-brush architecture |
| Ultrasonic spray | HOBOT-2S / S7 PRO | Fine mist designed to reduce over-wetting |
| Lowest observed entry price | DDC55 PRO | Observed around €189–€200 |
| Mature corded value | HOBOT-2S | Official $299 sale price with long cable and UPS |
| Large continuous panes | W2 PRO OMNI / C1 Station | Station-based workflows and long operating envelopes |

Key buying variables

Pane geometry: framed, frameless, segmented, corner-heavy or floor-to-ceiling.

Access: interior, exterior, balcony, skylight or overhead glass.

Operator model: occasional homeowner use vs. repeated facility-maintenance cycles.

Consumables: pad replacement, detergent, water tank and nozzle maintenance.

Safety: tether length, anchor point, emergency hold time and station anchoring.

Economics: purchase price is only one part of total cost; labor displacement and setup time can dominate.

ECOVACS WINBOT W2 PRO OMNI

Product overview and engineering position

Best overall for large residential glass and difficult-to-reach panes; strongest combination of station, autonomy, safety and workflow convenience.

| Parameter | Reported value |
| --- | --- |
| Observed price | $499.99 US list/current official price; €449 on ECOVACS Germany |
| Dimensions | 271 × 271 × 77 mm |
| Weight | 1.6 kg robot; 5.5 kg station |
| Adhesion | 5,500 Pa max (2,800 Pa while moving) |
| Spray / wetting | 6 nozzles; 60 ± 5 ml tank; microfiber pad |
| Navigation | WIN-SLAM 4.0; frame/edge detection; 7 modes |
| Battery | 4,500 mAh; up to 110 min; 30 min power-loss hold |

Source basis: ECOVACS official W2 PRO OMNI product page; ECOVACS Germany product page — https://www.ecovacs.com/us/shop/winbot-window-cleaning-robot/winbot-w2-pro-omni

ECOVACS WINBOT W2 PRO OMNI — Detailed Specifications

| Specification | Reported value |
| --- | --- |
| Robot size | 271 × 271 × 77 mm |
| Robot mass | 1.6 ± 0.1 kg |
| Station mass | 5.5 ± 0.2 kg |
| Max suction | 5,500 ± 500 Pa |
| Suction while moving | 2,800 Pa |
| Battery | 4,500 mAh / 24 V |
| Runtime | 110 ± 10 min |
| Power-off protection | 30 min |
| Tank | 60 ± 5 ml |
| Cleaning speed | ~1 min 45 s/m² in fast mode |
| Fast-mode area | ~55 m² |
| Deep-mode area | ~22 m² |
| Minimum window | 30 × 40 cm |
| Min. glass thickness | 3 mm |
| Noise | 63 dB minimum |
| Route planning | WIN-SLAM 4.0 |
| Cleaning modes | 7 |
| Cord | 5.5 m DC; automatic take-up |

Source basis: ECOVACS official W2 PRO OMNI product page; ECOVACS Germany product page — https://www.ecovacs.com/us/shop/winbot-window-cleaning-robot/winbot-w2-pro-omni

ECOVACS WINBOT W2 PRO OMNI — Functional Exploded View

Conceptual decomposition for engineering discussion; internal geometry and component count may differ from production hardware.

ECOVACS WINBOT W2 PRO OMNI — Engineering Evaluation

Best overall for large residential glass and difficult-to-reach panes; strongest combination of station, autonomy, safety and workflow convenience.

Engineering strengths

brushless blower

WIN-SLAM path planning

floating cleaning plate

automatic cord take-up

12-stage protection

Primary limitations / watch-outs

Confirm actual regional price and SKU.

Verify minimum pane geometry.

Inspect pad / nozzle consumables.

Use tether and supervision for exterior glass.

Treat manufacturer performance numbers as test-condition claims.

Source basis: ECOVACS official W2 PRO OMNI product page; ECOVACS Germany product page — https://www.ecovacs.com/us/shop/winbot-window-cleaning-robot/winbot-w2-pro-omni

HOBOT S7 PRO

Product overview and engineering position

Interesting for active scrubbing and ultrasonic mist, but less compelling than newer station-equipped competitors when speed, noise and edge finish dominate.

| Parameter | Reported value |
| --- | --- |
| Observed price | Observed 2026 market price about €322–€399; price varies by region |
| Dimensions | 262 × 264 × 89 mm |
| Weight | about 1.7–3.0 kg depending on source/market configuration |
| Adhesion | 4,800 Pa |
| Spray / wetting | Dual ultrasonic spray; about 52 ml tank in independent 2026 test |
| Navigation | AI / gyro-assisted route planning; Z/N patterns; app + remote |
| Battery | about 20 min normal backup; ~26–27 min measured emergency hold in 2026 test |

Source basis: HOBOT official S7 Pro page; CHIP 2026 independent test — https://www.hobot.com.tw/hobot-s7/

HOBOT S7 PRO — Detailed Specifications

| Specification | Reported value |
| --- | --- |
| Robot size | 262 × 264 × 89 mm |
| Mass | ~1.7 kg in CHIP 2026 test |
| Max suction | 4,800 Pa |
| Cleaning motion | Dual reciprocating pads; 600/min |
| Stroke | 4 mm |
| Spray | Dual ultrasonic, 15 μm |
| Tank | 52 ml in CHIP 2026 test |
| Power cable | 6.4 m in CHIP test |
| Safety line | 4.3 m in CHIP test |
| Control | App + remote |
| Noise | 31.8 sone in CHIP test |
| Emergency hold | ~26 min 34 s measured |
| Route patterns | Z / N / spot / manual |
| Station | None |

Source basis: HOBOT official S7 Pro page; CHIP 2026 independent test — https://www.hobot.com.tw/hobot-s7/

HOBOT S7 PRO — Functional Exploded View

Conceptual decomposition for engineering discussion; internal geometry and component count may differ from production hardware.

HOBOT S7 PRO — Engineering Evaluation

Interesting for active scrubbing and ultrasonic mist, but less compelling than newer station-equipped competitors when speed, noise and edge finish dominate.

Engineering strengths

dual reciprocating cleaning pads

600 strokes/min

4 mm pad stroke

ultrasonic 15 μm mist

app + remote control

Primary limitations / watch-outs

Confirm actual regional price and SKU.

Verify minimum pane geometry.

Inspect pad / nozzle consumables.

Use tether and supervision for exterior glass.

Treat manufacturer performance numbers as test-condition claims.

Source basis: HOBOT official S7 Pro page; CHIP 2026 independent test — https://www.hobot.com.tw/hobot-s7/

HOBOT-2S

Product overview and engineering position

Best value among the mature ultrasonic-spray designs if a corded workflow is acceptable.

| Parameter | Reported value |
| --- | --- |
| Observed price | $299 official US sale price (listed from $349) |
| Dimensions | 240 × 240 × 86 mm |
| Weight | 1.3 kg |
| Adhesion | 6.0 kgf adhesive force (manufacturer specification) |
| Spray / wetting | Dual ultrasonic 108 kHz; 15 μm mist; 2 × 26 ml tanks |
| Navigation | AI Technology V4; gyro; four edge-leakage sensors; zigzag / N-way |
| Battery | 20 min UPS / emergency hold |

Source basis: HOBOT USA official product page — https://hobot.us/products/hobot-2s-window-cleaning-robot-with-dual-ultrasonic-water-spray

HOBOT-2S — Detailed Specifications

| Specification | Reported value |
| --- | --- |
| Robot size | 240 × 240 × 86 mm |
| Mass | 1.3 kg |
| Power | 72 W |
| Suction force | 6.0 kgf |
| Spray | 2 × ultrasonic, 108 kHz |
| Mist size | 15 μm |
| Tanks | 2 × 26 ml |
| Minimum pane | 40 × 40 cm |
| Noise | 64 dB @ 1 m |
| Cleaning speed | ~2.4 min/m² |
| Cable | ~5 m |
| UPS | 20 min |
| Software | AI Technology V4 + gyro |
| Sensors | 4 edge-leakage sensors |
| Control | Bluetooth app + remote |

Source basis: HOBOT USA official product page — https://hobot.us/products/hobot-2s-window-cleaning-robot-with-dual-ultrasonic-water-spray

HOBOT-2S — Functional Exploded View

Conceptual decomposition for engineering discussion; internal geometry and component count may differ from production hardware.

HOBOT-2S — Engineering Evaluation

Best value among the mature ultrasonic-spray designs if a corded workflow is acceptable.

Engineering strengths

square form factor

ultrasonic spray

edge-leakage sensors

Bluetooth app + remote

long mains cable

Primary limitations / watch-outs

Confirm actual regional price and SKU.

Verify minimum pane geometry.

Inspect pad / nozzle consumables.

Use tether and supervision for exterior glass.

Treat manufacturer performance numbers as test-condition claims.

Source basis: HOBOT USA official product page — https://hobot.us/products/hobot-2s-window-cleaning-robot-with-dual-ultrasonic-water-spray

HUTT DDC55 PRO

Product overview and engineering position

Budget-oriented option with surprisingly broad surface compatibility, but slower and less precise at corners than premium track-based designs.

| Parameter | Reported value |
| --- | --- |
| Observed price | Observed around €189–€200 / about CHF 180 at launch; regional pricing varies |
| Dimensions | compact body; roughly 1–1.1 kg in current listings |
| Weight | about 1.1 kg in a 2026 commercial listing |
| Adhesion | 5,400 Pa manufacturer claim |
| Spray / wetting | 80 ml water tank; pulsed spray |
| Navigation | HUTTHINK variable-frequency algorithm; AI sensors; obstacle avoidance |
| Battery | manufacturer/market listings report emergency retention around 30 min |

Source basis: HUTT Wisdom official page; Galaxus 2026 independent review — https://www.huttwisdom.com/pages/hutt-ddc55-pro

HUTT DDC55 PRO — Detailed Specifications

| Specification | Reported value |
| --- | --- |
| Suction | 5,400 Pa manufacturer claim |
| Tank | 80 ml |
| Drive | Independent dual-motor drive |
| Mop architecture | 3D floating dual disks 2.0 |
| Air pressure response | 0.04 s manufacturer claim |
| Safety rope | 148 kg-rated claim |
| Emergency retention | 30 min manufacturer claim |
| Independent review speed | ~17 min for a >4 m² reference window |
| Independent review noise | 74.6 dB |
| Independent review | Good value; slow; corner limitations |
| Surfaces tested | Glass, mirrors, tiles, some frameless applications |

Source basis: HUTT Wisdom official page; Galaxus 2026 independent review — https://www.huttwisdom.com/pages/hutt-ddc55-pro

HUTT DDC55 PRO — Functional Exploded View

Conceptual decomposition for engineering discussion; internal geometry and component count may differ from production hardware.

HUTT DDC55 PRO — Engineering Evaluation

Budget-oriented option with surprisingly broad surface compatibility, but slower and less precise at corners than premium track-based designs.

Engineering strengths

dual-motor drive

3D floating dual disks 2.0

80 ml tank

0.04 s air-pressure compensation

148 kg-rated safety rope

Primary limitations / watch-outs

Confirm actual regional price and SKU.

Verify minimum pane geometry.

Inspect pad / nozzle consumables.

Use tether and supervision for exterior glass.

Treat manufacturer performance numbers as test-condition claims.

Source basis: HUTT Wisdom official page; Galaxus 2026 independent review — https://www.huttwisdom.com/pages/hutt-ddc55-pro

DREAME C1

Product overview and engineering position

Strong value proposition when corner coverage, app control and promotional pricing matter more than a portable station.

| Parameter | Reported value |
| --- | --- |
| Observed price | Observed €199 promotional price in Italy; €399 stated recommended retail price |
| Dimensions | 270 × 270 × 80.5 mm |
| Weight | 1.6 kg |
| Adhesion | 5,500 Pa max; 3,300 Pa maintained during normal operation |
| Spray / wetting | Dual spray; 80 ml tank; up to 60 min continuous wet cleaning |
| Navigation | AI path planning; Z/N selection; >10 sensors; edge detection |
| Battery | corded robot; C1 Station variant has 7,800 mAh / up to 180 min |

Source basis: Dreame Global official C1 page; Comet current listing — https://global.dreametech.com/products/dreame-c1

DREAME C1 — Detailed Specifications

| Specification | Reported value |
| --- | --- |
| Robot size | 270 × 270 × 80.5 mm |
| Mass | 1.6 kg |
| Max suction | 5,500 Pa max; 3,300 Pa maintained |
| Tank | 80 ml |
| Wet-clean runtime | Up to 60 min |
| Cleaning modes | 5 |
| Route planning | Z / N |
| Sensors | 10+ high-precision sensors |
| Edge detection | ~2 mm obstacle threshold; 0.02 s edge sensing claim |
| Cord | 5.5 m composite cord |
| Safety cord load | 140 kg manufacturer claim |
| Noise | 63 dB(A) |
| App | Dreamehome |
| CornerClean | Four 360° brushes |

Source basis: Dreame Global official C1 page; Comet current listing — https://global.dreametech.com/products/dreame-c1

DREAME C1 — Functional Exploded View

Conceptual decomposition for engineering discussion; internal geometry and component count may differ from production hardware.

DREAME C1 — Engineering Evaluation

Strong value proposition when corner coverage, app control and promotional pricing matter more than a portable station.

Engineering strengths

CornerClean with four 360° brushes

dual-spray wet cleaning

auto-resume

5 cleaning modes

app control

Primary limitations / watch-outs

Confirm actual regional price and SKU.

Verify minimum pane geometry.

Inspect pad / nozzle consumables.

Use tether and supervision for exterior glass.

Treat manufacturer performance numbers as test-condition claims.

Source basis: Dreame Global official C1 page; Comet current listing — https://global.dreametech.com/products/dreame-c1

Comparative Performance Matrix

| Criterion | W2 PRO OMNI | S7 PRO | HOBOT-2S | DDC55 PRO | Dreame C1 |
| --- | --- | --- | --- | --- | --- |
| Adhesion / stability | ★★★★★ | ★★★★☆ | ★★★★☆ | ★★★★☆ | ★★★★★ |
| Navigation | ★★★★★ | ★★★★☆ | ★★★★☆ | ★★★☆☆ | ★★★★★ |
| Edge / corner strategy | ★★★★★ | ★★★☆☆ | ★★★★☆ | ★★☆☆☆ | ★★★★★ |
| Wet cleaning | ★★★★★ | ★★★★★ | ★★★★★ | ★★★☆☆ | ★★★★★ |
| Portability | ★★★★★ | ★★★☆☆ | ★★★☆☆ | ★★★★☆ | ★★★☆☆ |
| Price value | ★★★★☆ | ★★★☆☆ | ★★★★★ | ★★★★★ | ★★★★★ |
| Large-pane workflow | ★★★★★ | ★★★☆☆ | ★★★☆☆ | ★★★☆☆ | ★★★★★ |
| Engineering maturity | ★★★★★ | ★★★★☆ | ★★★★★ | ★★★★☆ | ★★★★☆ |

Ratings are an engineering synthesis of published specifications and independent observations, not laboratory benchmark scores.

Application Fit Matrix

| Application | W2 PRO | S7 PRO | 2S | DDC55 PRO | C1 |
| --- | --- | --- | --- | --- | --- |
| Apartment interior windows | Excellent | Good | Excellent | Good | Excellent |
| Large floor-to-ceiling panes | Excellent | Fair–Good | Good | Good | Excellent |
| Frameless shower glass | Excellent | Good | Good | Good | Excellent |
| Frequent exterior maintenance | Excellent | Fair | Good | Good | Excellent |
| High-rise exterior use | Only with proper tether/anchor & supervision | Same | Same | Same | Same |
| Corner-heavy panes | Good–Excellent | Good | Good | Fair | Excellent |
| Lowest-cost purchase | Fair | Fair | Good | Excellent | Excellent |
| Cordless / station workflow | Excellent | No station | No | No | C1 Station variant |

A consumer window robot should be supervised during operation, especially outdoors or above occupied areas.

Total Cost of Ownership and Maintenance

| Cost element | What to budget for | Engineering implication |
| --- | --- | --- |
| Robot purchase | Initial hardware | Usually the largest one-time cost |
| Cleaning pads | Consumable / washable | Pad saturation and wear affect streaking |
| Cleaning solution | Model-specific or compatible solution | Nozzle compatibility matters |
| Battery | Long-term replacement | UPS capacity degrades with age |
| Nozzles / tanks | Ultrasonic systems may require replacement | Clogging can reduce wetting performance |
| Labor | Setup, repositioning, inspection | Large continuous panes produce the best labor economics |
| Safety equipment | Tether, anchor, station | Never omit because the robot has software protection |
| Storage / transport | Station and cables | Portable stations reduce setup friction |

Economic observation

The most important productivity variable is not the robot's nominal square-meter-per-minute rate; it is the complete cycle time: attach → start → clean → recover → move to next pane → refill → inspect. A slower robot can still be attractive when it allows the operator to perform other work between panes.

Recommended Operating Procedure (SOP)

Inspect the glass: remove loose grit, stickers, protruding hardware and heavy contamination that could damage the pad.

Inspect the robot: pad condition, blower inlet, spray nozzles, cable, safety tether and backup battery.

Prepare the surface and cleaning fluid according to the manufacturer instructions.

Secure the safety tether to a verified anchor before attaching the robot to exterior glass.

Place the robot on clean glass and confirm stable adhesion before starting the route.

Run the appropriate route: fast for routine maintenance, deep/spot/edge modes for heavier contamination where available.

Monitor the first cleaning cycle on every new window geometry.

After completion, return the robot to a safe position, remove it according to the manufacturer's sequence and clean the pad.

Inspect corners and frame edges manually when a presentation-grade finish is required.

Record abnormal suction, repeated edge errors, cable faults or battery degradation for maintenance.

For facility use

Create a pre-use checklist, equipment log, battery-health record and tether inspection schedule. For high-rise façades, use a formal work-at-height risk assessment and an independent fall-protection system where required by local rules.

Engineering Recommendation

For a general-purpose purchasing decision, the ECOVACS WINBOT W2 PRO OMNI is the strongest all-around candidate in this five-model set because it combines a portable station, battery/mains operation, route planning, edge detection, automatic cable management and a mature multi-layer protection architecture. Its principal disadvantage is price and the additional bulk of the station.

For cost-sensitive deployment, the HUTT DDC55 PRO and HOBOT-2S are more economical starting points. The DDC55 PRO is particularly interesting when the user values broad surface compatibility and can tolerate slower cleaning. The HOBOT-2S is a better fit when ultrasonic mist and mature corded operation are more important.

The Dreame C1 is the most compelling alternative for users who care about corners, app control and current promotional pricing. The HOBOT S7 PRO is technically differentiated by its reciprocating pads and ultrasonic mist, but independent 2026 testing indicates that its speed/noise profile makes it harder to justify at premium pricing.

| Scenario | Recommended model | Second choice |
| --- | --- | --- |
| Best overall | ECOVACS W2 PRO OMNI | Dreame C1 Station |
| Best value | Dreame C1 | HOBOT-2S |
| Lowest observed price | HUTT DDC55 PRO | Dreame C1 |
| Ultrasonic spray specialist | HOBOT-2S | HOBOT S7 PRO |
| Large continuous glass | W2 PRO OMNI | C1 Station |

Sources and Reference Notes

ECOVACS W2 PRO OMNI — official US
https://www.ecovacs.com/us/shop/winbot-window-cleaning-robot/winbot-w2-pro-omni

ECOVACS W2 PRO OMNI — official Germany
https://www.ecovacs.com/de/shop/winbot-window-cleaning-robot/winbot-w2-pro-omni

HOBOT S7 Pro — official
https://www.hobot.com.tw/hobot-s7/

HOBOT S7 Pro — CHIP 2026 test
https://www.chip.de/test-kaufberatung/haushalt-garten/hobot-s7-pro-pr-080-im-test_36c04667-b7d6-4e44-a160-f3f242ef6997.html

HOBOT-2S — official US
https://hobot.us/products/hobot-2s-window-cleaning-robot-with-dual-ultrasonic-water-spray

HUTT DDC55 PRO — official
https://www.huttwisdom.com/pages/hutt-ddc55-pro

HUTT DDC55 PRO — Galaxus 2026 review
https://www.galaxus.at/en/page/an-inexpensive-and-incredibly-impressive-window-cleaning-robot-42225

Dreame C1 — official global
https://global.dreametech.com/products/dreame-c1

Dreame C1 — Comet current retail listing
https://www.comet.it/dreame-robot-lavavetri-c1-window-cleaner-DRM00045T-prdtt

Access date: 13 August 2026.

This report is intended for engineering/procurement screening. Prices, specifications, firmware and regional availability can change. Verify the exact SKU and local safety requirements before purchase or deployment.